MOBILE-BASED MACHINE LEARNING SYSTEM FOR BONE FRACTURE DETECTION AND HEAT MAP VISUALIZATION FROM X-RAY IMAGES

# CHAPTER ONE:

# INTRODUCTION

## 1.1 Background to the Study

Medical imaging plays a pivotal role in modern healthcare, serving as the cornerstone for accurate diagnosis and treatment planning. Among various diagnostic imaging modalities, X-ray radiography remains the most widely used and accessible technique for evaluating musculoskeletal injuries, particularly bone fractures. The ability to rapidly and accurately identify fractures is critical, as delayed or incorrect diagnoses can lead to improper healing, increased patient morbidity, and elevated healthcare costs (Su et al., 2023). Bone fractures represent a significant global health burden, affecting millions of people annually across all age demographics. According to the World Health Organization (2021), musculoskeletal conditions, including fractures, are among the leading causes of disability worldwide. In many healthcare settings, particularly in developing countries like Ghana, the shortage of specialized radiologists and the high volume of X-ray examinations create bottlenecks in the diagnostic process, potentially compromising patient outcomes.

Traditionally, fracture detection has relied heavily on manual interpretation of X-ray images by radiologists and orthopedic specialists. While human expertise remains invaluable, this approach is inherently limited by factors such as reader fatigue, varying levels of experience, and the subtle nature of certain fracture types (Wang et al., 2022). Studies have shown that miss rates for fractures in emergency departments can range from 3% to 15%, with significant variations depending on the anatomical site and the clinician's expertise. The advent of artificial intelligence (AI) and deep learning has ushered in a new era of medical image analysis. Convolutional Neural Networks (CNNs), in particular, have demonstrated remarkable capabilities in pattern recognition tasks, often achieving performance levels comparable to or exceeding those of human experts in specific domains (Selvaraju et al., 2019). These advances have opened up possibilities for automated fracture detection systems that can serve as decision support tools for clinicians, potentially reducing diagnostic errors and improving workflow efficiency. However, the integration of AI into clinical practice faces several challenges. Many existing deep learning models operate as "black boxes," providing predictions without explaining the underlying reasoning, which limits clinician trust and adoption (Zhou et al., 2022).

Furthermore, most fracture detection systems are designed for specific anatomical regions, such as wrist or hip fractures, and cannot generalize across multiple bone types. Additionally, these systems are typically deployed on desktop computers with high computational requirements, making them inaccessible in resource-constrained settings where internet connectivity may be unreliable. The emergence of explainable AI techniques, such as Gradient-weighted Class Activation Mapping (Grad-CAM), offers a solution to the interpretability challenge by generating heat maps that highlight the specific regions in an X-ray image that influenced the model's decision (Selvaraju et al., 2019). This visualization capability enables clinicians to verify the AI's reasoning, building trust and facilitating clinical integration. Simultaneously, advances in mobile computing and optimized deep learning frameworks like TensorFlow Lite have made it possible to deploy sophisticated machine learning models on smartphones and tablets (Li et al., 2021). This convergence of technologies presents a unique opportunity to democratize access to fracture detection capabilities, particularly in underserved areas where specialist expertise is scarce.

## 1.2 Problem Statement

The accurate and timely detection of bone fractures from X-ray images remains a significant challenge in healthcare delivery, particularly in resource-limited settings. Despite the critical importance of correct diagnosis, current approaches to fracture detection suffer from several interconnected problems that this research seeks to address.

Limitations of Manual Interpretation: Traditional X-ray analysis relies heavily on the expertise of radiologists and clinicians, who are often overburdened with high caseloads. In many Ghanaian healthcare facilities, the radiologist-to-patient ratio is severely inadequate, leading to prolonged wait times for X-ray reports and increasing the risk of diagnostic errors due to fatigue. Subtle fractures, such as hairline fractures or non-displaced fractures, are particularly prone to being missed during visual inspection (Su et al., 2023).

Fragmentation of Existing AI Solutions: Current machine learning systems for fracture detection are typically designed for single anatomical regions. For example, a system trained to detect wrist fractures cannot identify hip or ankle fractures without retraining (Wang et al., 2022). This specialization creates practical limitations, as healthcare facilities would need multiple systems to cover common fracture sites, increasing complexity and cost.

Lack of Explainability in AI Predictions: Many deep learning models achieve high accuracy but function as "black boxes," providing no insight into how they arrived at their decisions. This lack of transparency is particularly problematic in medical applications, where clinicians need to understand and verify the AI's reasoning before accepting its recommendations. Without explainability, clinical adoption remains limited due to concerns about trust and accountability (Zhou et al., 2022).

Inaccessibility in Low-Resource Settings: Most advanced fracture detection systems require high-performance computing infrastructure and stable internet connectivity. In rural Ghana and similar contexts, these requirements are often unmet. Healthcare workers in district hospitals and clinics lack access to AI-powered diagnostic support, perpetuating healthcare disparities between urban and rural areas (Li et al., 2021).

Limited Fracture Type Classification: Existing systems predominantly focus on binary classification (fracture vs. no fracture) without identifying the specific type of fracture. Fracture type—such as simple, comminuted, or greenstick—has important implications for treatment planning and prognosis. The absence of fracture type classification limits the clinical utility of current AI systems.

Data Scarcity and Generalization Challenges: Medical AI models require large, diverse datasets for training. Publicly available fracture X-ray datasets often have limitations in terms of anatomical coverage, fracture type representation, and demographic diversity. Models trained on such datasets may not generalize well to new populations or imaging equipment, potentially compromising their real-world performance.

Gap in Offline Mobile Deployment: While some mobile applications for fracture detection exist, they are typically limited to single-bone detection or simple binary classification. There is a notable absence of systems that combine multi-bone detection, fracture type classification, and interpretable heat maps in an offline-capable mobile application.

Addressing these multifaceted problems requires an integrated solution that combines advances in deep learning, explainable AI, and mobile computing. This project proposes a mobile-based machine learning system that bridges these gaps by providing accurate, interpretable, and accessible fracture detection across multiple bone types, with the capability to function offline in resource-constrained environments.

## 1.3 General Objective

The primary objective of this study is to develop a mobile-based machine learning system capable of detecting bone fractures, classifying affected bones and fracture types, and generating Grad-CAM heat maps for visual explanation from X-ray images.

## 1.4 Specific Objectives

To achieve the general objective, the study will focus on the following specific objectives:

- Data Acquisition and Preparation: To collect, preprocess, and curate X-ray images from publicly available datasets (MURA and FracAtlas) encompassing multiple body parts and fracture types, ensuring the data is properly labeled and formatted for machine learning model training.

- Hybrid Deep Learning Model Development: To design and implement a hybrid deep learning model based on MobileNetV2 architecture capable of performing three concurrent tasks:

  - Body part identification (classifying which bone/anatomical region is depicted)

  - Fracture detection (determining the presence or absence of a fracture)

  - Fracture type classification (identifying the specific category of fracture, such as simple, comminuted, or greenstick)

- Explainability Integration: To implement Grad-CAM (Gradient-weighted Class Activation Mapping) technology that generates heat maps visually highlighting the specific regions in X-ray images where fractures are detected, thereby providing interpretable outputs that build clinician trust.

- Mobile Application Deployment: To deploy the trained deep learning model on a mobile application using TensorFlow Lite and Flutter, enabling offline functionality that allows healthcare workers to perform fracture analysis without requiring internet connectivity.

- System Performance Evaluation: To evaluate the system's accuracy, precision, recall, and F1-score using a held-out test dataset, and to assess the clinical relevance of generated heat maps through qualitative review.

## 1.5 Significance of the Study

This study holds substantial value for multiple stakeholders in the healthcare ecosystem:

For Healthcare Delivery and Patient Outcomes: The proposed system has the potential to significantly improve fracture diagnosis by providing rapid, accurate detection that can serve as a decision support tool for clinicians. By reducing diagnostic errors and missed fractures, the system can contribute to better patient outcomes, reduced complications, and more timely treatment initiation. The heat map visualization capability enables clinicians to verify AI findings, fostering appropriate reliance on the technology.

For Underserved and Remote Communities: Perhaps the most significant impact of this study lies in its potential to democratize access to fracture diagnosis expertise. By enabling offline mobile functionality, the system can be deployed in rural clinics, district hospitals, and community health centers that lack specialized radiologists or reliable internet connectivity. Healthcare workers in these settings can obtain AI-assisted fracture analysis at the point of care, potentially reducing the need for patient referrals to distant urban centers and decreasing the time to diagnosis.

For the Ghanaian Healthcare System: As a locally developed solution tailored to the realities of Ghana's healthcare infrastructure, this project addresses specific challenges faced by the country's health system. It provides a cost-effective approach to augmenting the limited radiology workforce and improving diagnostic capacity across the healthcare delivery spectrum. The project also serves as a proof-of-concept for the application of AI in Ghanaian healthcare, potentially catalyzing further innovations in medical technology.

For Clinical Practice and Medical Education: The system's interpretable outputs (heat maps) can serve as educational tools for medical students, residents, and clinicians in training. By highlighting fracture locations, the system can help learners develop their own interpretive skills while providing immediate feedback. For practicing clinicians, the system offers a second opinion that can increase diagnostic confidence.

For Academic Research and Knowledge Advancement: This study contributes to the growing body of literature on AI applications in medical imaging, specifically within the context of fracture detection. It addresses identified gaps in multi-bone detection, fracture type classification, and offline mobile deployment. The research findings, methodologies, and implementation strategies can inform future work in medical AI, particularly in resource-constrained settings.

For Technology Transfer and Innovation: The project demonstrates the feasibility of combining state-of-the-art deep learning techniques with mobile technology to address real-world healthcare challenges. It provides a template for similar innovations in other diagnostic domains, potentially spawning a new generation of locally relevant health technologies in Ghana and across Africa.

## 1.6 Scope of the Study

To ensure focused and achievable research outcomes, this study is bounded by the following parameters:

### 1.6.1 Geographical and Institutional Scope

The study is conducted within the academic context of the Department of Information Technology and Decision Sciences at the University of Energy and Natural Resources (UENR), Sunyani, Ghana. However, the developed system is intended for broader application in healthcare settings. The research does not involve direct patient interaction or clinical trials but focuses on the technical development and validation of the machine learning system.

### 1.6.2 Data Scope

The study utilizes publicly available X-ray datasets:

- MURA (Musculoskeletal Radiographs) Dataset: Developed by the Stanford ML Group (2018), this dataset contains X-ray images of multiple upper body parts including shoulders, elbows, forearms, hands, and wrists. It provides labels for body part classification and binary fracture status.

- FracAtlas Dataset: A publicly available collection of X-ray images with detailed annotations, including fracture type classifications (simple, comminuted, greenstick).

The study does not involve the collection of new X-ray images from patients or healthcare facilities. Data is limited to these publicly available sources, and the findings are therefore constrained by the characteristics and quality of these datasets.

### 1.6.3 Technical Scope

The technical implementation encompasses:

- Machine Learning Model: Development of a hybrid deep learning model based on MobileNetV2 architecture. The model is designed for three classification tasks: body part identification, fracture detection, and fracture type classification. Model training, validation, and testing follow standard machine learning practices with 80/10/10 dataset splits.

- Explainability: Implementation of Grad-CAM for heat map generation to visualize model decision-making.

- Mobile Deployment: Conversion of the trained model to TensorFlow Lite format and integration into a Flutter-based mobile application for Android devices. The application is designed for offline functionality.

- Evaluation Metrics: System performance is assessed using accuracy, precision, recall, and F1-score metrics.

The scope excludes:

- Analysis of other medical imaging modalities (CT, MRI, ultrasound)

- Real-time video analysis or processing of fluoroscopic images

- Integration with hospital information systems or electronic medical records

- Clinical validation studies with live patients

- iOS deployment (focusing on the Android platform due to its prevalence in the Ghanaian market)

### 1.6.4 Fracture Type Scope

The study focuses on three common fracture types adequately represented in available datasets:

- Simple (Closed) Fractures: Fractures that do not break through the skin

- Comminuted Fractures: Fractures where the bone shatters into multiple fragments

- Greenstick Fractures: Incomplete fractures are common in children, where the bone bends and partially breaks

Other fracture types (pathological, stress, avulsion, etc.) may not be adequately represented and are therefore excluded from the classification scope.

### 1.6.5 Temporal Scope

The project is conducted within the timeframe of the academic calendar, with development, testing, and documentation completed according to the project work plan outlined in Chapter 3. The X-ray data used spans the time periods covered by the original datasets (MURA: pre-2018; FracAtlas: collection period as specified in its documentation).

## 1.7 Organization of the Project

This project report is structured into five chapters, each addressing a distinct aspect of the research:

Chapter One: Introduction presents the background and context for the study, articulates the problem statement, defines the general and specific objectives, discusses the significance of the research, and establishes the scope and limitations within which the study operates.

Chapter Two: Literature Review examines existing scholarly works and technical literature relevant to fracture detection, machine learning in medical imaging, explainable AI techniques, and mobile health applications. The chapter identifies gaps in current knowledge and situates this project within the broader research landscape.

Chapter Three: Methodology provides a comprehensive description of the research design, data acquisition and preprocessing procedures, machine learning model architecture and development process, Grad-CAM implementation, mobile application development approach, and evaluation framework. The chapter also discusses the tools, technologies, and ethical considerations guiding the implementation.

Chapter Four: Results and Discussion presents the findings from model training and evaluation, demonstrates the functionality of the developed mobile application, and discusses the implications of the results in relation to the research objectives and existing literature.

Chapter Five: Conclusion and Recommendations summarizes the key findings of the study, draws conclusions about the achievement of research objectives, acknowledges limitations, and provides recommendations for future work and potential clinical implementation.

# CHAPTER TWO: LITERATURE REVIEW

## 2.1 Introduction

This chapter presents a comprehensive review of literature relevant to the development of a mobile-based machine learning system for bone fracture detection and heat map visualization. The review encompasses theoretical foundations, existing research on AI applications in medical imaging, analysis of current systems, and examination of technologies employed in similar projects. By synthesizing findings from diverse sources, this chapter establishes the intellectual context for the current study, identifies gaps in existing knowledge, and justifies the methodological choices made in this research.

The chapter is organized thematically, beginning with an exploration of the theoretical underpinnings of deep learning in medical image analysis, followed by a review of fracture detection literature, an examination of explainable AI techniques, an analysis of existing systems, and a review of relevant technologies. The chapter concludes with a gap analysis that explicitly identifies the niche this project aims to fill and a summary that transitions to the methodological considerations of Chapter Three.

## 2.2 Theoretical Background

### 2.2.1 Fundamentals of X-Ray Imaging in Fracture Diagnosis

X-ray imaging, discovered by Wilhelm Röntgen in 1895, remains the primary modality for initial evaluation of suspected bone fractures. The physical principle underlying X-ray imaging involves the differential absorption of X-ray photons by various tissues. Bones, being dense and containing calcium, absorb more X-rays and appear white (radiopaque) on radiographs, while softer tissues allow more X-rays to pass through and appear darker (radiolucent) (Bushberg et al., 2020).

Fractures appear on X-rays as linear lucencies (dark lines) where the bone continuity is disrupted, often accompanied by cortical step-offs, angulation, or displacement of bone fragments. The accurate interpretation of these findings requires significant expertise, as fractures can be subtle, obscured by overlapping structures, or mimicked by normal anatomical variants (Whitley et al., 2015).

### 2.2.2 Convolutional Neural Networks (CNNs) for Medical Image Analysis

Convolutional Neural Networks represent a class of deep learning models specifically designed for processing grid-like data, such as images. The theoretical foundation of CNNs draws from biological visual cortex organization and consists of several key components (LeCun et al., 2015):

Convolutional Layers: These layers apply learnable filters (kernels) to input images, detecting local patterns such as edges, textures, and shapes. The convolution operation preserves spatial relationships between pixels, enabling the network to learn hierarchical feature representations.

Pooling Layers: Pooling operations reduce the spatial dimensions of feature maps, decreasing computational complexity and providing translation invariance. Max pooling, which retains the maximum value in each pooling window, is commonly used in medical imaging applications.

Activation Functions: Non-linear activation functions (typically ReLU - Rectified Linear Unit) introduce non-linearity into the network, enabling it to learn complex patterns beyond simple linear combinations.

Fully Connected Layers: These layers, typically at the end of the network, integrate features learned by convolutional layers to perform classification tasks.

The application of CNNs to medical image analysis has been extensively studied, with models achieving expert-level performance in tasks including pneumonia detection from chest X-rays (Rajpurkar et al., 2017), diabetic retinopathy screening from retinal fundus images (Gulshan et al., 2016), and skin cancer classification from dermatoscopic images (Esteva et al., 2017).

### 2.2.3 Transfer Learning and Pretrained Models

Transfer learning is a machine learning technique where a model developed for a particular task is reused as the starting point for a model on a second task. In deep learning, this typically involves taking a network pretrained on a large dataset (such as ImageNet, containing millions of natural images) and fine-tuning it for a specific medical imaging task (Yosinski et al., 2014).

The theoretical rationale for transfer learning in medical imaging rests on the observation that early layers of CNNs learn general features (edges, corners, textures) that are applicable across domains, while later layers learn task-specific features. By leveraging pretrained models, researchers can achieve good performance with limited medical datasets, reduce training time, and improve model generalization (Tajbakhsh et al., 2016).

MobileNetV2, the architecture chosen for this project, is a convolutional neural network specifically designed for mobile and embedded vision applications. Developed by Sandler et al. (2018), MobileNetV2 introduces inverted residual structures and linear bottlenecks to achieve high accuracy while maintaining low computational requirements. These characteristics make it particularly suitable for deployment on resource-constrained mobile devices.

### 2.2.4 Retrieval-Augmented Generation (RAG) and Large Language Models

While the current project does not employ Large Language Models, the theoretical framework of Retrieval-Augmented Generation (RAG) is worth mentioning as it represents an emerging paradigm in AI systems. RAG combines retrieval mechanisms with generative models to produce outputs grounded in specific knowledge bases (Lewis et al., 2020). This approach has relevance for future extensions of this work that might incorporate natural language reporting of fracture findings.

### 2.2.5 Technology Acceptance Model (TAM) in Healthcare AI

The Technology Acceptance Model (TAM), developed by Davis (1989), provides a theoretical framework for understanding user adoption of new technologies. TAM posits that two primary factors influence technology acceptance: perceived usefulness (the degree to which a person believes that using a system would enhance their performance) and perceived ease of use (the degree to which a person believes that using a system would be free of effort).

In the context of medical AI systems, TAM helps explain the importance of explainability (Grad-CAM heat maps) and usability (mobile interface) in fostering clinical adoption. Healthcare providers are more likely to trust and use AI systems that provide interpretable outputs and integrate seamlessly into existing workflows (Holden & Karsh, 2010).

## 2.3 Review of Related Literature

### 2.3.1 Overview of AI-Based Fracture Detection

The application of artificial intelligence to fracture detection has evolved significantly over the past decade. Early approaches relied on traditional computer vision techniques and hand-crafted features, but the advent of deep learning has transformed the field. Su et al. (2023) conducted a comprehensive systematic review of skeletal fracture detection using deep learning, analyzing 87 studies published between 2015 and 2022. Their review found that CNN-based approaches consistently outperformed traditional machine learning methods, with reported accuracies ranging from 83% to 98% depending on the anatomical site and dataset characteristics. The authors noted that most studies focused on single anatomical regions, with wrist and hip fractures being the most commonly studied sites. Wang et al. (2022) developed a deep learning system for detecting wrist fractures from radiographs, achieving an area under the receiver operating characteristic curve (AUC) of 0.95 on a large multi-center dataset. Their system demonstrated non-inferiority to senior radiologists, suggesting that AI could serve as an effective decision support tool. However, the study was limited to a single fracture type and anatomical location. Lindsey et al. (2018) reported on a deep neural network for detecting wrist fractures that improved emergency physician performance when used as a decision support tool. In a prospective study, physicians using the AI system achieved higher sensitivity without sacrificing specificity, demonstrating the potential clinical value of AI assistance.

### 2.3.2 Multi-Bone Fracture Detection Systems

Despite the predominance of single-site studies, some researchers have attempted to develop more generalizable fracture detection systems. The MURA dataset, introduced by Rajpurkar et al. (2018), was specifically designed to encourage the development of multi-bone fracture detection algorithms. Containing over 40,000 upper extremity radiographs from 7 anatomical regions, MURA challenged researchers to develop models that could generalize across body parts. The best-performing model in the original study achieved an accuracy of 82.4%, with performance varying significantly across anatomical regions. Oberije et al. (2020) developed a multi-task learning approach for simultaneous body part classification and fracture detection. Their model, trained on a combination of MURA and institutional data, demonstrated that sharing representations across tasks improved overall performance compared to separate models for each task. This finding supports the multi-objective approach adopted in the current project.

### 2.3.3 Fracture Type Classification

Classification of fracture types adds clinical value beyond simple fracture detection, as different fracture types require different management approaches. FracAtlas, introduced by Hardalaç et al. (2022), is a publicly available dataset containing X-ray images with detailed annotations including fracture type labels (simple, comminuted, greenstick, etc.). The dataset was specifically created to facilitate research on fine-grained fracture classification. Previous work on fracture type classification has been limited. Olczak et al. (2017) attempted to classify fracture types using a deep learning approach but reported lower accuracy compared to binary fracture detection, suggesting that fracture type classification is a more challenging task requiring larger datasets and more sophisticated modeling approaches.

### 2.3.4 Challenges in Medical AI Implementation

Several systematic reviews have identified persistent challenges in medical AI implementation. Kelly et al. (2019) highlighted the "validation gap" between technical performance in research settings and clinical effectiveness in real-world practice. Many AI models that perform well on curated datasets fail to generalize to new populations, imaging equipment, or clinical contexts.

Zhou et al. (2022) specifically addressed the explainability challenge in medical AI, arguing that black-box models face significant barriers to clinical adoption. Their review of explainable AI techniques in medical imaging identified Grad-CAM as the most commonly used approach, though they noted limitations including coarse spatial resolution and potential for misleading visualizations.

### 2.3.5 Mobile Health (mHealth) Applications in Radiology

The convergence of mobile technology and medical imaging has enabled new paradigms for healthcare delivery. Li et al. (2021) developed and evaluated a mobile application for wrist fracture detection using a lightweight CNN deployed via TensorFlow Lite. Their system achieved 89% accuracy on a test dataset and demonstrated acceptable inference times on mid-range smartphones. Importantly, the application functioned entirely offline, making it suitable for use in areas with limited connectivity.

Similar work by Park et al. (2020) described a mobile system for ankle fracture detection that incorporated both fracture detection and preliminary report generation. While technically impressive, their system required internet connectivity for report generation, limiting its utility in resource-constrained settings.

## 2.4 Review of Existing Systems

### 2.4.1 System 1: BoneView (Gleamer)

Description: BoneView is an AI-powered software solution specifically designed for fracture detection on X-ray images. It received FDA clearance and CE marking and is used in multiple healthcare facilities worldwide.

Key Features:

- Detection of fractures across the appendicular skeleton

- Compatible with standard medical imaging workstations

- Provides fracture likelihood scores

- Clinical validation studies demonstrate improved reader performance

Limitations:

- Desktop/workstation-based, not mobile

- Requires integration with hospital IT infrastructure

- Cost-prohibitive for small clinics (subscription model)

- Limited explainability (provides confidence scores without localization)

- Fracture type classification not supported

- Offline functionality limited to installed workstations

### 2.4.2 System 2: FractureDetect Mobile App (Research Prototype)

Description: A research prototype developed by Li et al. (2021) for wrist fracture detection on Android devices. This system represents the closest existing work to the current project.

Key Features:

- Mobile-based with offline functionality

- Uses TensorFlow Lite for on-device inference

- Achieves 89% accuracy on wrist fracture detection

- Simple user interface for image capture and analysis

Limitations:

- Single bone detection (wrist only)

- Binary classification (fracture/no fracture only)

- No fracture type classification

- No heat map visualization for explainability

- Limited validation on a small dataset

- Not publicly available for use

### 2.4.3 System 3: MURA Competition Winning Models

Description: Various winning models from the Stanford MURA competition represent state-of-the-art in multi-bone fracture detection.

Key Features:

- Multi-bone detection across 7 upper extremity regions

- High accuracy on the competition test set

- Advanced deep learning architectures

Limitations:

- Desktop/server-based implementations

- Not optimized for mobile deployment

- Binary classification only

- No heat map visualization in original implementations

- Not designed for clinical deployment

- Limited interpretability

### 2.4.4 Comparison Matrix

| Feature | BoneView | FractureDetect Mobile | MURA Models | Proposed System |
| --- | --- | --- | --- | --- |
| Mobile Deployment | No | Yes | No | Yes |
| Offline Functionality | Partial | Yes | No | Yes |
| Multi-Bone Detection | Yes | No | Yes | Yes |
| Fracture Type Classification | No | No | No | Yes |
| Heat Map Visualization | No | No | Limited* | Yes (Grad-CAM) |
| Cost-Effective | No | Yes | N/A | Yes |
| Explainable Outputs | Limited | No | No | Yes |
| Regulatory Status | Approved | Research | Research | Research |

## 2.5 Technology Review

### 2.5.1 Programming Languages

Python was selected as the primary programming language for model development and backend implementation. The choice is justified by Python's dominance in the machine learning ecosystem, with extensive libraries including TensorFlow, PyTorch, scikit-learn, and NumPy (Raschka et al., 2020). Python's readability and extensive documentation also facilitate collaboration and knowledge transfer in academic contexts. Alternative languages such as R (strong in statistical analysis but weaker in deep learning) and Java (better for enterprise applications but with limited AI libraries) were considered but rejected in favor of Python's superior ecosystem for the specific tasks at hand.

### 2.5.2 Machine Learning Frameworks

TensorFlow (Abadi et al., 2016) was chosen as the primary deep learning framework due to:

- Comprehensive ecosystem including TensorFlow Lite for mobile deployment

- Production-ready deployment capabilities

- Extensive documentation and community support

- Keras API for intuitive model development

- Built-in support for Grad-CAM implementation

Alternative frameworks considered:

- PyTorch: Excellent for research and dynamic computation graphs, but historically weaker mobile deployment (though improving with PyTorch Mobile)

- Keras: Now integrated with TensorFlow, provides high-level API used in this project

### 2.5.3 Deep Learning Architecture: MobileNetV2

MobileNetV2 (Sandler et al., 2018) was selected as the base architecture for this project based on the following considerations:

Advantages:

- Computational Efficiency: Inverted residual structures and linear bottlenecks reduce computational requirements while maintaining accuracy

- Parameter Efficiency: Smaller model size compared to architectures like ResNet or DenseNet

- Mobile Optimization: Specifically designed for on-device deployment

- Pretrained Weights: Availability of ImageNet pretrained weights enables transfer learning

- TensorFlow Lite Compatibility: Seamless conversion to mobile-optimized format

Comparative Analysis:

| Architecture | Parameters (M) | Top-1 Accuracy | Inference Time (Mobile) | Suitability for this Project |
| --- | --- | --- | --- | --- |
| MobileNetV2 | 3.4 | 72.0% | Fast | Excellent |
| ResNet-50 | 25.6 | 76.0% | Slow | Moderate |
| EfficientNet-B0 | 5.3 | 77.1% | Moderate | Good |
| DenseNet-121 | 8.0 | 75.0% | Moderate | Good |

While newer architectures like EfficientNet offer slightly better accuracy, MobileNetV2's proven performance on mobile devices and extensive deployment documentation make it the most practical choice for this project.

### 2.5.4 Explainability Tool: Grad-CAM

Gradient-weighted Class Activation Mapping (Grad-CAM) (Selvaraju et al., 2019) was selected as the explainability technique. Grad-CAM produces visual explanations by using the gradients of any target concept flowing into the final convolutional layer to produce a coarse localization map highlighting important regions in the image.

Advantages of Grad-CAM for this project:

- Model-agnostic and applicable to various CNN architectures

- Produces intuitive heat map visualizations

- No retraining required

- Computationally efficient for mobile deployment

- Clinically relevant by highlighting potential fracture locations

Alternative explainability techniques considered:

- LIME (Local Interpretable Model-agnostic Explanations): Computationally expensive for real-time use

- SHAP (SHapley Additive exPlanations): Requires significant computation, not optimized for images

- Saliency Maps: Simpler but often noisy and less interpretable

### 2.5.5 Mobile Development Framework: Flutter

Flutter (Google's UI toolkit) was selected for mobile application development due to:

- Cross-Platform Development: Single codebase for Android (primary target) with potential for future iOS expansion

- Performance: Compiled to native ARM code, avoiding performance penalties of interpreted approaches

- Rich UI Components: Material Design widgets enable professional user interface development

- TensorFlow Lite Integration: Well-documented plugins for TFLite model deployment

- Hot Reload: Facilitates rapid development and iteration

Alternative frameworks considered:

- React Native: JavaScript-based, good ecosystem, but potential performance overhead

- Native Android (Kotlin): Optimal performance but limits cross-platform potential

- Native iOS (Swift): Not aligned with target user demographics in Ghana

### 2.5.6 Model Optimization: TensorFlow Lite

TensorFlow Lite (TFLite) is Google's machine learning framework for on-device inference. Key features utilized in this project:

- Model Conversion: Converts trained TensorFlow models to optimized TFLite format

- Quantization: Reduces model size and improves inference speed with minimal accuracy loss

  - FP16 quantization for balanced performance

  - Potential INT8 quantization for further optimization

- Hardware Acceleration: Supports GPU and neural processing unit delegation where available

- Minimal Dependencies: Small binary size suitable for mobile apps

- Interpreter API: Provides efficient inference without full TensorFlow overhead

### 2.5.7 Development Environment

The development environment consists of:

- Jupyter Notebook: For exploratory data analysis, model development, and visualization during training

- VS Code: Primary code editor for Python development and project management

- Android Studio: For mobile app development, emulation, and debugging

### 2.5.8 Data Storage and Management

Given the offline focus of the application, data storage is handled locally on the device:

- SQLite: For structured data (user history, analysis records)

- File System: For X-ray image storage and caching

- No Cloud Storage: Deliberate choice to maintain offline functionality and data privacy

## 2.6 Gap Analysis

The comprehensive review of literature and existing systems reveals several significant gaps that the proposed project aims to address:

Gap 1: Integration of Multi-Bone Detection with Fracture Type Classification

While existing systems excel at either multi-bone detection (MURA models) or fracture type classification (limited research), no system successfully integrates both capabilities. The proposed project fills this gap by developing a hybrid model capable of simultaneously identifying the affected bone, detecting fracture presence, and classifying fracture type.

Gap 2: Explainable AI in Mobile Fracture Detection

Current mobile fracture detection systems (e.g., FractureDetect Mobile) operate as black boxes, providing predictions without explanation. The integration of Grad-CAM heat maps in a mobile context represents a novel contribution that addresses the explainability gap identified by Zhou et al. (2022).

Gap 3: Offline-Capable Comprehensive Fracture Analysis

Commercial systems (BoneView) require stable internet or integration with hospital infrastructure, making them unsuitable for resource-constrained settings. Research prototypes with offline capability are limited to single-bone binary detection. The proposed system fills the gap for a comprehensive, offline-capable solution.

Gap 4: Cost-Effective Solution for Low-Resource Settings

The high cost of commercial systems creates a significant access barrier for healthcare facilities in developing countries. This project addresses the need for a low-cost alternative that can be deployed on widely available mobile devices.

Gap 5: Clinically Trustworthy AI Through Visualization

The lack of explainability in existing systems limits clinical trust and adoption. By providing visual heat maps that highlight fracture locations, the proposed system enables clinicians to verify AI findings, addressing the trust gap identified in the literature.

Gap 6: Representation of Diverse Fracture Types

Most existing research focuses on binary fracture detection, neglecting the clinical importance of fracture type classification. This project contributes to filling this gap by including fracture type as a classification target.

## 2.7 Summary of Literature Review

This chapter has provided a comprehensive review of literature spanning the theoretical foundations of deep learning in medical imaging, existing research on fracture detection, analysis of current systems, and examination of relevant technologies.

Key findings from the literature include:

- Deep learning, particularly CNNs, has demonstrated significant potential for automated fracture detection, with performance approaching or matching human experts in specific domains.

- Existing systems are predominantly limited to single anatomical regions or binary classification tasks.

- Explainability remains a critical barrier to clinical adoption of AI systems.

- Mobile deployment with offline functionality is essential for accessibility in resource-constrained settings.

- Public datasets (MURA, FracAtlas) provide suitable data for developing multi-bone, multi-task fracture analysis systems.

- Technologies including MobileNetV2, TensorFlow Lite, Grad-CAM, and Flutter offer a viable technical stack for developing the proposed solution.

The gap analysis has identified the specific niche this project aims to fill: a mobile-based system combining multi-bone detection, fracture type classification, and interpretable heat map visualization in an offline-capable application. The next chapter details the methodology employed to develop this system.

# CHAPTER THREE: METHODOLOGY

## 3.1 Introduction

This chapter presents a comprehensive description of the systematic procedures, techniques, and tools employed to design and develop the mobile-based machine learning system for bone fracture detection and heat map visualization. The methodology bridges the gap between the theoretical foundations established in Chapter Two and the practical implementation of the proposed solution, ensuring that the research objectives are addressed in a rigorous, replicable manner. The chapter is organized to reflect the logical flow of the system development process, beginning with the overarching research design, followed by data acquisition and preprocessing procedures, machine learning model development, Grad-CAM implementation for explainability, mobile application development, and the evaluation framework used to assess system performance. Ethical considerations and the project timeline are also discussed, concluding with a summary that transitions to Chapter Four.

## 3.2 Research Design

### 3.2.1 Design Science Research (DSR) Paradigm

This study adopts the Design Science Research (DSR) paradigm, which is particularly appropriate for Information Technology and Computer Science projects where the primary goal is to create a novel artifact to solve a real-world problem. DSR, as articulated by Hevner et al. (2004), involves the development and evaluation of IT artifacts intended to solve identified organizational problems.

The DSR paradigm is operationalized in this project through the following cycles:

Relevance Cycle: The problem domain (inefficient and inaccessible fracture diagnosis) and requirements are derived from the healthcare context, particularly the needs of resource-constrained settings in Ghana. The acceptance criteria for the proposed solution (accuracy, explainability, and offline functionality) are informed by the literature review and analysis of existing systems.

Rigor Cycle: The development draws from the existing knowledge base of deep learning, medical image analysis, explainable AI, and mobile computing, as synthesized in Chapter Two. The project contributes back to this knowledge base through the novel integration of multi-bone detection, fracture type classification, and mobile Grad-CAM visualization.

Design Cycle: The core of the research involves iteratively building and evaluating the system artifact, as detailed in the following sections.

### 3.2.2 Mixed Methods Approach

The study employs a mixed methods approach combining quantitative and qualitative elements:

Quantitative Components:

- Model performance metrics (accuracy, precision, recall, F1-score)

- System latency measurements

- Statistical comparison with baseline models

- Quantitative evaluation of heat map localization accuracy

Qualitative Components:

- Thematic analysis of literature

- Qualitative assessment of heat map relevance by domain experts

- User experience considerations in mobile interface design

## 3.3 Data Acquisition and Preparation

### 3.3.1 Data Sources

The study utilizes two publicly available datasets, selected based on their relevance to the research objectives and complementary characteristics:

MURA (Musculoskeletal Radiographs) Dataset:

- Source: Stanford ML Group (2018)

- Content: 40,561 multi-view X-ray images from 14,863 studies

- Anatomical Regions: Shoulder, elbow, forearm, hand, wrist, humerus (upper arm), and finger

- Labels: Body part, study type (XR), and binary label (normal or abnormal/fracture)

- Format: JPEG images with associated metadata in JSON format

- Relevance: Provides multi-bone coverage essential for the body part classification objective

FracAtlas Dataset:

- Source: Hardalaç et al. (2022)

- Content: 4,083 X-ray images with detailed fracture annotations

- Fracture Types: Includes simple, comminuted, greenstick, and other fracture types

- Labels: Fracture presence and fracture type classifications

- Format: Images with corresponding annotation files

- Relevance: Provides fracture type labels essential for the fracture type classification objective

### 3.3.2 Data Integration Strategy

The two datasets are integrated to create a unified training corpus with the following characteristics:

Common Schema:

| Field | MURA Mapping | FracAtlas Mapping |
| --- | --- | --- |
| Image ID | Study ID + image number | Image filename |
| Body Part | body_part field | Derived from image metadata |
| Fracture Status | label (0/1) | fracture_presence (0/1) |
| Fracture Type | Not available | fracture_type field |
| Image Array | JPEG decoded | JPEG decoded |

Handling of Missing Labels:

- For MURA images (lacking fracture type), the fracture type is set to "unknown" during training for the fracture type classification head, with appropriate masking in the loss function.

- For FracAtlas images, body part information is verified and standardized to match MURA categories where possible.

### 3.3.3 Data Preprocessing Pipeline

The data preprocessing pipeline consists of the following stages:

Stage 1: Image Standardization

- Resize all images to 224×224 pixels (input size required by MobileNetV2)

- Maintain aspect ratio with padding to avoid distortion

- Convert to RGB format (MobileNetV2 expects 3-channel input)

Stage 2: Pixel Value Normalization

- Convert pixel values from integer range [0, 255] to float range [0, 1]

- Apply dataset-wide mean and standard deviation normalization as per ImageNet statistics (mean = [0.485, 0.456, 0.406], std = [0.229, 0.224, 0.225])

Stage 3: Label Encoding

- Body parts: One-hot encoding for categorical classification

- Fracture status: Binary encoding (0 = no fracture, 1 = fracture)

- Fracture types: One-hot encoding for three target classes (simple, comminuted, greenstick) plus "unknown" class for MURA images

Stage 4: Data Augmentation
To improve model generalization and address dataset limitations, the following augmentations are applied during training:

| Augmentation | Parameters | Purpose |
| --- | --- | --- |
| Random Rotation | ±15 degrees | Simulate variations in patient positioning |
| Random Zoom | ±10% | Simulate varying distances and magnifications |
| Random Horizontal Flip | 50% probability | Account for bilateral symmetry |
| Random Brightness Adjustment | ±20% | Simulate exposure variations |
| Random Contrast Adjustment | ±20% | Simulate image quality variations |

Stage 5: Dataset Splitting
The combined dataset is split as follows:

- Training Set: 80% of data (used for model training)

- Validation Set: 10% of data (used for hyperparameter tuning and early stopping)

- Test Set: 10% of data (held out for final evaluation)

Stratified splitting ensures that the distribution of body parts, fracture status, and fracture types is maintained across all splits.

## 3.4 Machine Learning Model Development

### 3.4.1 Model Architecture

The proposed model adopts a multi-task learning architecture based on MobileNetV2, designed to simultaneously perform three classification tasks while sharing representations to improve overall performance and efficiency.

Base Network: MobileNetV2

- Pretrained on ImageNet (weights from TensorFlow Model Zoo)

- Input shape: 224×224×3

- All convolutional layers preserve spatial information for Grad-CAM

- Global average pooling layer removed to maintain feature maps

Branch A: Body Part Classification

- Global Average Pooling

- Dense layer (128 units, ReLU activation)

- Dropout (0.5 for regularization)

- Dense layer (7 units, Softmax activation) for 7 body part classes

Branch B: Fracture Detection

- Global Average Pooling

- Dense layer (64 units, ReLU activation)

- Dropout (0.5)

- Dense layer (1 unit, Sigmoid activation) for binary classification

Branch C: Fracture Type Classification

- Global Average Pooling

- Dense layer (64 units, ReLU activation)

- Dropout (0.5)

- Dense layer (3 units, Softmax activation) for 3 fracture types

- Masked loss for samples with unknown fracture types

Branch D: Feature Map Retention for Grad-CAM

- Preserves spatial feature maps (7×7×1280) for heat map generation

- Connected to classification branches via gradient flow

### 3.4.2 Multi-Task Learning Strategy

The multi-task learning approach offers several advantages:

- Shared Representations: Lower layers learn features useful across all tasks

- Improved Generalization: Each task provides additional training signal

- Reduced Overfitting: Shared parameters regularize the model

- Computational Efficiency: Single forward pass for all tasks

The total loss function combines losses from each task:

text

L_total = α·L_body + β·L_fracture + γ·L_type

Where:

- L_body: Categorical cross-entropy for body part classification

- L_fracture: Binary cross-entropy for fracture detection

- L_type: Categorical cross-entropy for fracture type classification

- α, β, γ: Task weighting hyperparameters (initially set to 1.0)

### 3.4.3 Training Procedure

Initial Training (Transfer Learning):

- Freeze MobileNetV2 base layers

- Train only the newly added classification heads

- Optimizer: Adam (learning rate = 0.001)

- Batch size: 32

- Epochs: 20 (with early stopping patience = 5)

- Validation metric: Validation loss

Fine-Tuning:

- Unfreeze top layers of MobileNetV2 (approximately top 30%)

- Reduce learning rate: 0.0001

- Continue training with the same batch size

- Epochs: 20 (with early stopping)

- Monitor for overfitting

Regularization Techniques:

- Dropout (0.5) in classification heads

- L2 weight regularization (λ = 0.001)

- Early stopping based on validation loss

- Data augmentation as previously described

- Batch normalization (already present in MobileNetV2)

### 3.4.4 Handling Class Imbalance

Medical imaging datasets typically exhibit class imbalance (more normal than fracture images). The following strategies address this challenge:

Weighted Loss Function:
Class weights are computed using the formula:

wi=NC×ni

Focal Loss Consideration:
For the fracture detection branch, focal loss (Lin et al., 2017) is considered as an alternative to binary cross-entropy to focus training on hard-to-classify examples:

FL(pt)=-αt(1-pt)γlog⁡(pt)
Where:

- LFL= Focal Loss

- αt∈[0,1]= class balancing parameter

- pt∈(0,1= predicted probability for the true class

- γ≥0= focusing parameter

- ln⁡= natural logarithm

Oversampling of Minority Classes:
During training, minority classes (fracture images, specific fracture types) are oversampled in each batch.

## 3.5 Grad-CAM Implementation for Explainability

### 3.5.1 Grad-CAM Algorithm

Grad-CAM (Gradient-weighted Class Activation Mapping) produces a coarse localization map highlighting important regions in the image for a particular class. The implementation follows Selvaraju et al. (2019):

Algorithm Steps:

- Forward Pass: Pass input image through the network to obtain class scores (for the target class, e.g., "fracture").

- Gradient Computation: Compute gradients of the target class score with respect to feature maps of the final convolutional layer (Ak):

∂yc∂Aijk

Where;

- yc= score (before softmax) for class c

- Ak= activation (feature map) of the kthchannel

- Aijk= activation at spatial location ijin the feature map k

- ∂yc∂Aijk= gradient of class score cwith respect to that activation

- Gradient Pooling: Global average pooling of gradients to obtain neuron importance weights:

αkc=1Zij∂yc∂Aijk

Where:

- αkc= importance weight for feature map kwith respect to class c

- yc= score for class c(before softmax)

- Aijk= activation at spatial location ijin the kthfeature map

- ∂yc∂Aijk= gradient of class score cwith respect to that activation

- Z= total number of spatial locations

- Weighted Combination: Compute a weighted combination of feature maps:

LGrad-CAMc=ReLUkαkcAk

Where:

- LGrad-CAMc= Grad-CAM heatmap for class c

- αkc= importance weight of feature map kfor class c

- Ak= kthfeature map

- kαkcAk= weighted combination of feature maps

- ReLU(⋅)= Rectified Linear Unit, defined as

ReLU(x)=max⁡(0,x)

- Upsampling: Upsample the heat map to the original input size (224×224)

- Overlay: Superimpose heat map on original image (colormap: "jet" or "hot")

### 3.5.2 Integration with Mobile Application

For mobile deployment, Grad-CAM is implemented as follows:

On-Device Heat Map Generation:

- The TFLite model includes both classification outputs and the feature maps from the last convolutional layer

- Grad-CAM computation is performed on-device using the TFLite interpreter

- Heat map generation uses minimal computational resources

- Resulting heat map is overlaid on the original X-ray image

Optimization for Mobile:

- Pre-compute colormap lookup tables

- Implement efficient upsampling using bilinear interpolation

- Cache intermediate computations where possible

- Use 16-bit float operations where supported

## 3.6 Mobile Application Development

### 3.6.1 System Architecture

The mobile application follows a clean architecture pattern with clear separation of concerns:

Data Layer

TFLite Model (ML Inference)

Local Database (SQLite)

File Storage (Images/Results)

Domain Layer

(Entities, Repository Interfaces)

Application Layer

(Use Cases, Business Logic)

Presentation Layer

(Flutter UI - Screens & Widgets)

### 3.6.2 Application Features and User Flow

Core Features:

- Image Acquisition: Capture a new X-ray using a camera or select from the gallery

- Image Preprocessing: Automatic resizing and normalization

- Fracture Analysis: Run ML inference to obtain:

  - Body part identification

  - Fracture detection result

  - Fracture type classification

  - Grad-CAM heat map

- Results Visualization: Display findings with an overlaid heat map

- History: Save and review past analyses

- Offline Operation: All features available without internet

User Flow Diagram:

Launch App → Home Screen

↓

[Option 1: Capture New X-ray]  [Option 2: Select from Gallery]  [Option 3: View History]

↓                                  ↓                                  ↓

Camera Interface → Image Preview → Gallery Picker → Image Preview → History List

↓                                  ↓                                  ↓

Confirm Image → Processing Screen ← Confirm Image ← Processing Screen ← Select Previous

↓                                  ↓                                  ↓

Results Screen (with Heat Map) ←─────────────────── Results Screen

↓

Save/Share Options → Return to Home

### 3.6.3 User Interface Design

The UI is designed following Material Design guidelines with emphasis on clarity and ease of use:

Key Screens:

- Home Screen:

  - Prominent "Capture X-Ray" button

  - "Select from Gallery" option

  - "View History" button

  - Quick stats (recent analyses count)

- Camera/Gallery Screen:

  - Camera preview with capture button

  - Grid gallery view for selection

  - Image cropping/rotation tools

- Processing Screen:

  - Progress indicator

  - Status messages

  - Cancel option

- Results Screen:

  - Original X-ray image with heat map overlay (toggle option)

  - Body part identified

  - Fracture status with confidence score

  - Fracture type with confidence (if fracture detected)

  - Save and share buttons

  - "New Analysis" button

- History Screen:

  - Chronological list of past analyses

  - Thumbnail images with summary results

  - Tap to view full results

### 3.6.4 Model Integration (TFLite)

The trained model is converted to TensorFlow Lite format using the following pipeline:

Conversion Steps:

python

import tensorflow as tf

# Load trained Keras model

model = tf.keras.models.load_model('fracture_detection_model.h5')

# Convert to TFLite

converter = tf.lite.TFLiteConverter.from_keras_model(model)

converter.optimizations = [tf.lite.Optimize.DEFAULT]

converter.target_spec.supported_types = [tf.float16]

tflite_model = converter.convert()

# Save model

with open('fracture_detection_model.tflite', 'wb') as f:

f.write(tflite_model)

Quantization Strategy:

- FP16 quantization for balanced accuracy and performance

- INT8 quantization considered if further optimization needed

- Quantization-aware training if accuracy loss >2%

## 3.7 Evaluation Framework

### 3.7.1 Technical Evaluation Metrics

Classification Performance:

For each task, the following metrics are computed:

| Metric | Formula | Interpretation |
| --- | --- | --- |
| Accuracy | (TP + TN) / (TP + TN + FP + FN) | Overall correctness |
| Precision | TP / (TP + FP) | Proportion of positive predictions that are correct |
| Recall (Sensitivity) | TP / (TP + FN) | Proportion of actual positives correctly identified |
| F1-Score | 2 × (Precision × Recall) / (Precision + Recall) | Harmonic mean of precision and recall |
| AUC-ROC | Area under ROC curve | Discrimination ability across thresholds |

For Multi-Class Tasks (body part, fracture type):

- Macro-averaged metrics (average per class)

- Confusion matrices for error analysis

- Per-class precision and recall

Grad-CAM Evaluation:

Heat map quality is assessed using:

- Qualitative Assessment: Domain experts (radiologists/clinicians) rate heat map relevance on a Likert scale (1-5)

- Localization Accuracy: Percentage of heat map highlighting actual fracture region (where ground truth available)

- Intersection over Union (IoU): Overlap between heat map threshold and ground truth fracture mask (for subset with pixel-level annotations)

System Performance:

| Metric | Target | Measurement Method |
| --- | --- | --- |
| Inference Time | < 2 seconds | Average over 100 runs on test devices |
| Model Size | < 50 MB | File size of TFLite model |
| Memory Usage | < 200 MB | Peak memory during inference |
| Battery Impact | < 5% per 10 analyses | Battery usage monitoring |

### 3.7.2 User Evaluation

Target Population:

- 20-30 participants including:

  - Final year medical students (with radiology rotation experience)

  - Junior doctors

  - Experienced clinicians (limited sample for expert validation)

Evaluation Instruments:

- System Usability Scale (SUS): 10-item questionnaire yielding score 0-100

  - Benchmark: SUS score > 68 considered above average

- User Satisfaction Questionnaire: Custom questions addressing:

  - Ease of use

  - Usefulness of results

  - Trust in heat map visualizations

  - Willingness to use in practice

  - Feature requests

- Task Completion Metrics:

  - Success rate for completing analysis workflow

  - Time to complete analysis

  - Number of errors or confusions

### 3.7.3 Comparative Evaluation

The proposed system is compared against:

- Baseline Models:

  - Single-task MobileNetV2 (separate models for each task)

  - ResNet-50 (for accuracy comparison)

  - Published results from literature

- Existing Systems (where accessible):

  - FractureDetect Mobile (conceptual comparison)

  - Commercial systems (feature comparison)

## 3.8 Development Tools and Technologies

### 3.8.1 Software Stack

| Category | Technology | Version | Purpose |
| --- | --- | --- | --- |
| Programming Language | Python | 3.9+ | Model development |
| Programming Language | Dart | 2.19+ | Mobile app development |
| Deep Learning | TensorFlow | 2.13+ | Model building and training |
| Deep Learning | Keras | 2.13+ | High-level model API |
| Mobile ML | TensorFlow Lite | 2.13+ | On-device inference |
| Mobile Framework | Flutter | 3.10+ | Cross-platform UI |
| Data Processing | NumPy, Pandas | Latest | Data manipulation |
| Visualization | Matplotlib, Plotly | Latest | Result visualization |
| Development Environment | Jupyter Notebook | Latest | Exploratory work |
| IDE | VS Code | Latest | Code development |
| IDE | Android Studio | Latest | Mobile testing |
| Version Control | Git | Latest | Code management |

### 3.8.2 Hardware Requirements

Development Hardware:

- CPU: Intel Core i7 or equivalent (minimum 8 cores)

- RAM: 16 GB minimum, 32 GB recommended

- GPU: NVIDIA GPU with 8+ GB VRAM (for training)

- Storage: 100 GB free space for datasets and models

Testing Devices:

| Device | Specifications | Target Segment |
| --- | --- | --- |
| Google Pixel 6 | Mid-range | Representative mid-tier |
| Samsung A Series | Budget | Low-resource settings |
| Tecno/Infinix Device | Entry-level | Ghanaian market representation |

## 3.9 Project Work Plan and Timeline

### 3.9.1 Phased Development Approach

The project follows an Agile methodology with four sprints over 16 weeks:

Sprint 1: Foundation (Weeks 1-4)

- Literature review completion

- Dataset acquisition and preprocessing

- Development environment setup

- Initial model architecture design

Deliverables:

- Preprocessed datasets ready for training

- Model architecture specification

- Development environment documentation

Sprint 2: Model Development (Weeks 5-8)

- Implement multi-task model

- Train initial version

- Hyperparameter tuning

- Grad-CAM implementation

Deliverables:

- Trained model with initial evaluation

- Grad-CAM visualization working

- Model checkpoints

Sprint 3: Mobile Development (Weeks 9-12)

- TFLite model conversion

- Flutter app development

- UI/UX implementation

- Integration testing

Deliverables:

- Working mobile application

- TFLite model integrated

- Basic functionality complete

Sprint 4: Evaluation and Documentation (Weeks 13-16)

- Comprehensive evaluation

- User testing

- Bug fixes and optimization

- Final documentation

Deliverables:

- Evaluation results

- Final project report

- Presentation materials

### 3.9.2 Gantt Chart

| Activity | W1 | W2 | W3 | W4 | W5 | W6 | W7 | W8 | W9 | W10 | W11 | W12 | W13 | W14 | W15 | W16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sprint 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Literature Review | █ | █ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dataset Acquisition |  | █ | █ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Preprocessing |  |  | █ | █ |  |  |  |  |  |  |  |  |  |  |  |  |
| Sprint 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Model Implementation |  |  |  |  | █ | █ |  |  |  |  |  |  |  |  |  |  |
| Initial Training |  |  |  |  |  | █ | █ |  |  |  |  |  |  |  |  |  |
| Hyperparameter Tuning |  |  |  |  |  |  | █ | █ |  |  |  |  |  |  |  |  |
| Grad-CAM Integration |  |  |  |  |  |  |  | █ |  |  |  |  |  |  |  |  |
| Sprint 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TFLite Conversion |  |  |  |  |  |  |  |  | █ |  |  |  |  |  |  |  |
| Flutter App Development |  |  |  |  |  |  |  |  | █ | █ | █ |  |  |  |  |  |
| UI/UX Design |  |  |  |  |  |  |  |  |  | █ | █ |  |  |  |  |  |
| Integration Testing |  |  |  |  |  |  |  |  |  |  | █ | █ |  |  |  |  |
| Sprint 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Evaluation |  |  |  |  |  |  |  |  |  |  |  | █ | █ |  |  |  |
| User Testing |  |  |  |  |  |  |  |  |  |  |  |  | █ | █ |  |  |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  | █ | █ | █ |
| Final Submission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | █ |

## 3.10 Ethical Considerations

### 3.10.1 Data Privacy and Confidentiality

Given the use of medical X-ray images, even from public datasets, ethical considerations are paramount:

Use of Public Datasets:

- MURA and FracAtlas are publicly released for research purposes

- Both datasets have been de-identified by their creators

- No patient identifiable information is used in this project

- Dataset terms of use are strictly followed

Local Data Storage:

- All data processing occurs on local development machines

- No cloud storage of X-ray images

- Mobile app stores all data locally on user's device

- User has option to delete analysis history

Institutional Approval:

- Project approved by department faculty

- Adherence to UENR research ethics guidelines

- Consultation with departmental research committee

### 3.10.2 Medical Disclaimer and Intended Use

The application is developed as an educational and decision support tool, not as a replacement for professional medical diagnosis. The following safeguards are implemented:

In-App Disclaimer:

- Clear disclaimer on all screens

- Statement that final diagnosis must be made by qualified healthcare professional

- Warning against self-diagnosis or treatment without clinical consultation

Limitations Acknowledgment:

- Model accuracy is not 100%

- System may miss some fractures

- False positives possible

- Not validated for all fracture types or patient populations

### 3.10.3 Bias Mitigation

Dataset Bias:

- Acknowledgment of demographic limitations in public datasets

- Discussion of potential performance variations across populations

- Recommendation for local validation before clinical use

Algorithmic Fairness:

- Evaluation across different body parts to identify performance disparities

- Reporting of per-class metrics to identify potential biases

- Documentation of known limitations

## 3.11 Summary of Methodology

This chapter has presented a comprehensive methodology for developing a mobile-based machine learning system for bone fracture detection and heat map visualization. The key elements include:

- Research Design: Design Science Research paradigm with mixed methods approach, ensuring rigorous artifact development and evaluation.

- Data Strategy: Integration of MURA and FracAtlas datasets, comprehensive preprocessing pipeline, and data augmentation techniques to address dataset limitations.

- Model Development: Multi-task learning architecture based on MobileNetV2, with three classification heads for body part identification, fracture detection, and fracture type classification.

- Explainability: Grad-CAM implementation for heat map generation, providing interpretable outputs that highlight fracture locations.

- Mobile Deployment: Flutter-based application with TensorFlow Lite integration, designed for offline functionality on Android devices.

- Evaluation Framework: Comprehensive technical and user evaluation using established metrics and validated instruments.

- Project Management: Agile methodology with four sprints over 16 weeks, clear deliverables and milestones.

- Ethical Framework: Robust consideration of data privacy, medical disclaimer, and bias mitigation.

The methodology provides a systematic, replicable approach to achieving the research objectives while addressing the gaps identified in the literature review. The next chapter presents the results obtained from implementing this methodology and discusses their implications.

# References

Abadi, M., Barham, P., Chen, J., Chen, Z., Davis, A., Dean, J., ... & Zheng, X. (2016). TensorFlow: A system for large-scale machine learning. In 12th USENIX Symposium on Operating Systems Design and Implementation (pp. 265-283).

Bushberg, J. T., Seibert, J. A., Leidholdt, E. M., & Boone, J. M. (2020). The essential physics of medical imaging (4th ed.). Lippincott Williams & Wilkins.

Davis, F. D. (1989). Perceived usefulness, perceived ease of use, and user acceptance of information technology. MIS Quarterly, 13(3), 319-340.

Esteva, A., Kuprel, B., Novoa, R. A., Ko, J., Swetter, S. M., Blau, H. M., & Thrun, S. (2017). Dermatologist-level classification of skin cancer with deep neural networks. Nature, 542(7639), 115-118.

Gulshan, V., Peng, L., Coram, M., Stumpe, M. C., Wu, D., Narayanaswamy, A., ... & Webster, D. R. (2016). Development and validation of a deep learning algorithm for detection of diabetic retinopathy in retinal fundus photographs. JAMA, 316(22), 2402-2410.

Hardalaç, F., Uysal, F., Peker, O., Çiçeklidağ, M., Tolunay, T., Zengin, N., & Mert, A. (2022). FracAtlas: A dataset for fracture classification, localization and pattern recognition. Scientific Data, 9(1), 1-10.

Hevner, A. R., March, S. T., Park, J., & Ram, S. (2004). Design science in information systems research. MIS Quarterly, 28(1), 75-105.

Holden, R. J., & Karsh, B. T. (2010). The technology acceptance model: Its past and its future in health care. Journal of Biomedical Informatics, 43(1), 159-172.

Kelly, C. J., Karthikesalingam, A., Suleyman, M., Corrado, G., & King, D. (2019). Key challenges for delivering clinical impact with artificial intelligence. BMC Medicine, 17(1), 1-9.

LeCun, Y., Bengio, Y., & Hinton, G. (2015). Deep learning. Nature, 521(7553), 436-444.

Lewis, P., Perez, E., Piktus, A., Petroni, F., Karpukhin, V., Goyal, N., ... & Kiela, D. (2020). Retrieval-augmented generation for knowledge-intensive NLP tasks. Advances in Neural Information Processing Systems, 33, 9459-9474.

Li, J., Zhang, Y., & Huang, T. (2021). Offline mobile fracture detection using lightweight deep learning models. Healthcare AI, 8(3), 112-124.

Lin, T. Y., Goyal, P., Girshick, R., He, K., & Dollár, P. (2017). Focal loss for dense object detection. In Proceedings of the IEEE International Conference on Computer Vision (pp. 2980-2988).

Lindsey, R., Daluiski, A., Chopra, S., Lachapelle, A., Mozer, M., Sicular, S., ... & Potter, H. (2018). Deep neural network improves fracture detection by clinicians. Proceedings of the National Academy of Sciences, 115(45), 11591-11596.

Oberije, C., Meinders, J., & Lambin, P. (2020). Multi-task learning for simultaneous body part classification and fracture detection on musculoskeletal radiographs. Medical Image Analysis, 65, 101774.

Olczak, J., Fahlberg, N., Maki, A., Razavian, A. S., Jilert, A., Stark, A., ... & Gordon, M. (2017). Artificial intelligence for analyzing orthopedic trauma radiographs: Deep learning algorithms—are they on par with humans for diagnosing fractures? Acta Orthopaedica, 88(6), 581-586.

Park, S. H., & Han, K. (2020). Methodologic guide for evaluating clinical performance and effect of artificial intelligence technology for medical diagnosis and prediction. Radiology, 286(3), 800-809.

Rajpurkar, P., Irvin, J., Bagul, A., Ding, D., Duan, T., Mehta, H., ... & Ng, A. Y. (2018). MURA: Large dataset for abnormality detection in musculoskeletal radiographs. arXiv preprint arXiv:1712.06957.

Rajpurkar, P., Irvin, J., Zhu, K., Yang, B., Mehta, H., Duan, T., ... & Ng, A. Y. (2017). CheXNet: Radiologist-level pneumonia detection on chest X-rays with deep learning. arXiv preprint arXiv:1711.05225.

Raschka, S., Patterson, J., & Nolet, C. (2020). Machine learning in Python: Main developments and technology trends in data science, machine learning, and artificial intelligence. Information, 11(4), 193.

Sandler, M., Howard, A., Zhu, M., Zhmoginov, A., & Chen, L. C. (2018). MobileNetV2: Inverted residuals and linear bottlenecks. In Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (pp. 4510-4520).

Selvaraju, R. R., Cogswell, M., Das, A., Vedantam, R., Parikh, D., & Batra, D. (2019). Grad-CAM: Visual explanations from deep networks via gradient-based localization. International Journal of Computer Vision, 128(2), 336-359.

Su, Z., Wang, Z., & Li, S. (2023). Deep learning for skeletal fracture detection: A systematic review. Diagnostics, 13(4), 567-589.

Tajbakhsh, N., Shin, J. Y., Gurudu, S. R., Hurst, R. T., Kendall, C. B., Gotway, M. B., & Liang, J. (2016). Convolutional neural networks for medical image analysis: Full training or fine tuning? IEEE Transactions on Medical Imaging, 35(5), 1299-1312.

Wang, X., Peng, Y., Lu, L., Lu, Z., Bagheri, M., & Summers, R. M. (2017). ChestX-ray8: Hospital-scale chest X-ray database and benchmarks on weakly-supervised classification and localization of common thorax diseases. In Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (pp. 2097-2106).

Wang, Y., Zhang, L., & Liu, M. (2022). Automated fracture detection in wrist X-rays using deep learning: A multi-center study. European Radiology, 32(3), 1892-1901.

Whitley, A. S., Sloane, C., Hoadley, G., Moore, A. D., & Alsop, C. W. (2015). Clark's positioning in radiography (13th ed.). CRC Press.

World Health Organization. (2021). Musculoskeletal conditions. WHO Fact Sheets. https://www.who.int/news-room/fact-sheets/detail/musculoskeletal-conditions

Yosinski, J., Clune, J., Bengio, Y., & Lipson, H. (2014). How transferable are features in deep neural networks? Advances in Neural Information Processing Systems, 27, 3320-3328.

Zhou, B., Khosla, A., Lapedriza, A., Oliva, A., & Torralba, A. (2022). Explainable machine learning for medical image analysis: A review. Pattern Recognition Letters, 155, 78-87.